# Documentation for the H₂-DRI-EAF Optimization Model

## 1. Introduction

This documentation provides a comprehensive guide to using the H₂-DRI-EAF optimization model developed in GAMS.
The model is a linear optimization framework representing the planning and operation of a renewable-centric hydrogen-based steelmaking system. It features hourly resolution over an entire year, simulating a flexible and decarbonized Electrolytic Hydrogen–Direct Reduced Iron–Electric Arc Furnace (H₂-DRI-EAF) process.

Key elements of the model include:

- Energy sources: Solar PV and wind power are the primary sources, supplemented by regional grid electricity.

- Storage systems: Lithium-ion batteries for electricity storage; hydrogen tanks for hydrogen storage; DRI storage.

- Production systems:

  - Alkaline electrolyzers produce hydrogen.

  - A DRI furnace processes iron ore using hydrogen and natural gas.

  - The Electric Arc Furnace (EAF) melts a flexible mix of DRI and scrap steel to produce crude steel.

- Storage flexibility: Hot DRI can be stored as cold DRI (HBI) with an associated energy penalty.

- Demand matching: Crude steel is continuously cast to meet hourly steel demand.

The objective of the model is to minimize total annualized system costs, optimizing both capacity planning and hourly operational dispatch, subject to technological, resource, and policy constraints.

## 2. Software Installation and Setup

### 2.1 Install GAMS

- Download GAMS:
Visit the official website: https://www.gams.com/download/

- Select the appropriate version for your operating system (Windows, macOS, Linux).

- Install GAMS following the on-screen instructions.

- (Optional but recommended) Set Environment Variables:
After installation, you can add the GAMS directory (e.g., C:\GAMS\win64\xx) to your system PATH to easily run GAMS from any command line.

- License Requirement:
GAMS requires a valid license file for full functionality.

- A free demo license is included with the download, but it has size and functionality limitations (e.g., maximum number of variables and constraints).

- For large-scale optimization or commercial/academic projects, you may need to purchase a license separately.

- Information on licensing options is available at https://www.gams.com/buy/.

- If you already have a license, place the license file (gamslice.txt) in your GAMS installation directory.

### 2.2 Setting Up the Model Files

- Obtain the model files (provided in the project repository or via direct download).

- The typical structure includes:

  - model.gms: Main model file

  - DRI_EAF_input.xlsx: Excel file containing input datasets

  - steel_output.xlsm: Output file (after running)

- Place all files in the same working directory.

- Open GAMS Studio or any compatible GAMS editor.

### 2.3 Running the Model

- Open run_model.gms in GAMS Studio.

- Check that the correct GDX input files are linked.

- Press F9 (Run Selected File) or F8 (Run Entire Project) to start the model run.

- Once completed, output files including steel_output.xlsm will be generated in the working directory.

## 3. Model Details

The objective function minimizes the total annualized costs of the renewable-dominated H₂-DRI-EAF system, encompassing annuitized capital costs, equipment operation and maintenance (O&M) costs, raw material costs, external energy costs, and labor costs (Equations (1)-(3)). Fixed costs for each component are calculated as the product of planning capacity and the corresponding annuity, which accounts for annuitized capital costs and fixed O&M costs (Equation (4)). For batteries, fixed costs include components related to both power capacity and energy capacity (Equation (5)). Variable costs are determined by the product of time-varying variables (e.g., hourly natural gas consumption) and the corresponding unit costs (Equation (6)).

| totc=fixctot+varctot |  | (1) |  |
| --- | --- | --- | --- |
| fixctot=fixcele+fixcely+fixcbat+fixctank+fixcdrif+fixceaf |  | (2) |  |
| varctot=varcio+varcscr+varcgp+varcng+varclab+varcom |  | (3) |  |
| fixci=capi·ANNCi | i∈(ele, ely,tank,drif,eaf) |  | (4) |
| fixcbat=capbatp·ANNCbatp+capbate·ANNCbate |  | (5) |  |
| varcj=tjvart,j·UNITCj | j∈(io,scr,gp,ng,lab,om) |  | (6) |

Balance constraints: Balance constraints are applied to electricity, hydrogen, DRI, and steel to ensure system equilibrium. For electricity, hydrogen, and DRI, these constraints state that generation plus net discharge must equal the respective demand (Equations (7)-(9)). For steel, special considerations are required since semi-finished and finished products are prone to oxidation without proper precautions, limiting their storage duration. Steel balance is modeled based on periodic contracts, assuming that total steel production within a period must meet the prorated demand over the period (Equation (10)).

| ele_gent,solar+ele_gent,wind+bat_outt-bat_int+gridt=ely_pt+dri_pt+drip_pt+eaf_pt | (7) |
| --- | --- |
| h2_gent+h2_outt-h2_int=dri_ht | (8) |
| dri_gent,h2+dri_gent,gas+dri_outt-dri_int=eaf_drit | (9) |
| t∈g0st_gent=t∈g0ST_DEt | (10) |

Capacity Constraints: Capacity constraints represent that the load of each facility should remain within feasible operational limits. For renewables, generation per unit capacity must not exceed the maximum output curves determined by weather conditions (Equations (11)-(12)). For batteries, charging and discharging power are restricted by power capacity, while SOC must remain within the energy capacity (Equations (13)-(15)). Electrolyzer arrays operate within a feasible output range, bounded by upper and lower limits (Equations (16)-(17)). Hydrogen storage tanks are subject to both upper SOC constraints a minimum SOC serving as a cushion (Equations (18)-(19)). For the DRI furnace, if flexibility is enabled, its load should stay within specified upper and lower boundaries (Equations (20)-(22)). Otherwise, the DRI furnace operates at full load continuously (Equation (23)). Steel output from the EAF is constrained to be less than or equal to its capacity on an hourly basis (Equation (24)). These constraints ensure reliable operation across all system components.

| ele_gent,solar≤capsolar·MAXGENt,solar | (11) |
| --- | --- |
| ele_gent,wind≤capwind·MAXGENt,wind | (12) |
| bat_outt≤capbatp | (13) |
| bat_int≤capbatp | (14) |
| bat_soct≤capbate | (15) |
| ely_pt≤capely·MAX_LOADely | (16) |
| ely_pt≥capely·MIN_LOADely | (17) |
| h2_soct≤captank | (18) |
| h2_soct≥captank·MIN_SOCh2 | (19) |
| dri_gent,h2+dri_gent,gas=dri_gent,tot | (20) |
| dri_gent,tot≤capdrif·MAX_LOADdri | (21) |
| dri_gent,tot≥capdrif·MIN_LOADdri | (22) |
| dri_gent,tot=capdrif | (23) |
| st_gent≤capeaf | (24) |

Storage constraints: Storage constraints define the relationship between the SOC and charge-discharge actions over the time series. The SOC in the current time slice equals the SOC from the previous time slice plus the net charge within the time step. For batteries, efficiency losses during charging and discharging are accounted for, while losses are neglected for hydrogen or DRI storage (Equations (25)-(27)).

| bat_soct=bat_soct-1-bat_outt/BAT_ηout+bat_int×BAT_ηin | (25) |
| --- | --- |
| h2_soct=h2_soct-1-h2_outt+h2_int | (26) |
| dri_soct=dri_soct-1-dri_outt+dri_int | (27) |

Ramp Constraints: Ramp constraints define the hourly ramp-up and ramp-down limits based on technological requirements. For hydrogen and natural gas, equations are formulated to approximate hourly changes in gas composition, avoiding non-linearity. In the DRI furnace, the hourly consumption of either hydrogen or natural gas is subject to rate-of-change limits, ensuring smooth operational adjustments (Equations (28)-(31)). The maximum share of natural gas in the DRI furnace is capped at 30% (Equation (32)). Similarly, changes in the DRI furnace load must adhere to specified ramp-up and ramp-down rates (Equations (33)-(34)).

| dri_gent-1,h2-dri_gent,h2≤capdrif×RAMPdri_gas_comp | (28) |
| --- | --- |
| dri_gent,h2-dri_gent-1,h2≤capdrif×RAMPdri_gas_comp | (29) |
| dri_gent-1,gas-dri_gent,gas≤capdrif×RAMPdri_gas_comp | (30) |
| dri_gent,gas-dri_gent-1,gas≤capdrif×RAMPdri_gas_comp | (31) |
| dri_gent,gas≤(drigent,gas+drigent,h2)×MAX_GAS_COMP | (32) |
| dri_gent,tot-dri_gent-1,tot≤capdrif×RAMP_UPdri_load | (33) |
| dri_gent-1,tot-dri_gent,tot≤capdrif×RAMP_DOWNdri_load | (34) |

Response Cycle Constraints: Response cycle constraints are developed to analyze the sensitivity of system responsiveness, particularly for DRI gas composition, DRI load, and grid power. Within each response cycle, the corresponding variable remains constant but may vary across cycles (Equations (35)-(37)). For DRI gas composition and DRI load, these constraints can replace ramp constraints. The ramp constraints are applied for hourly responsiveness while response cycle constraints are used for longer-duration responsiveness. This substitution allows the system to adapt to varying operational strategies.

| dri_gent,gas=dri_geng1,gas | t∈g1 | (35) |
| --- | --- | --- |
| dri_gent,tot=dri_geng2,tot | t∈g2 | (36) |
| gridt=gridg3 | t∈g3 | (37) |

Stoichiometry Balance: Stoichiometry balance broadly refers to ensuring the consistency of the input-output relationship in the system and maintaining proportionality across mass, energy, or other resources. For electrolyzers, hydrogen output per hour is directly proportional to the electrical input (Equation (38)). In the DRI furnace, inputs such as natural gas, hydrogen, electricity, and iron ore are connected to DRI output, with distinct stoichiometric data for natural gas-based and hydrogen-based DRI production (Equations (39)-(42)). Similarly, in the EAF, electricity consumption is distinguished between scrap-based and DRI-based production (Equations (44)-(45)). The energy penalty for reheating DRI is considered (Equation (43)). If flexibility in the scrap blend ratio is enabled, the scrap-to-steel ratio should follow the annual balance ratio only (Equation (47)). Otherwise, the scrap-to-steel ratio is fixed and remains constant for every hour (Equation (46)).

| h2_gent×STOICHele,h2=ely_pt | (38) |
| --- | --- |
| dri_gent,h2×STOICHh2,dri=dri_ht | (39) |
| dri_gent,gas×STOICHgas,dri=dri_gt | (40) |
| dri_ht×STOICHele,dri=dri_pt | (41) |
| (dri_gent,h2+dri_gent,gas)×STOICHio,dri=dri_iot | (42) |
| dri_outt×STOICHele,dri_reheat=drip_pt | (43) |
| eaf_drit×STOICHele,eaf_dri+eaf_scrt×STOICHele,eaf_scr=eaf_pt | (44) |
| eaf_drit/STOICHeaf_dri,st+eaf_scrt/STOICHeaf_scr,st=st_gent | (45) |
| eaf_scrt=st_gent×SCR_R | (46) |
| teaf_scrt=tst_gent×SCR_R | (47) |

Emission Constraints: Emission constraints indicate that the annual CO₂ emissions should be bounded by the exogenous emission caps (Equation (49)). CO₂ emissions are calculated based on contributions from natural gas and grid power, determined by multiplying their respective consumption with their emission factors (Equation (48)). Also, the emission cap constraints can be relaxed with a carbon tax policy, where emission costs are equal to total CO₂ emissions multiplied by the exogenous carbon tax and should be involved in the objective function (Equations (50)-(51)).

| emit,tot=gridt·EFgp+dri_gt·EFng | (48) |
| --- | --- |
| temit,tot≤EMI_CAP | (49) |
| temit,tot≤EMI_CAP+EMI_REL | (50) |
| emic=temit,tot·CT | (51) |

Non-served Demand Introduction: For testing the robustness across different weather years under scenarios without grid power, the non-served demand is introduced to replace the grid power in the electricity balance constraints, as shown in Equation (52). Meanwhile, the penalty of non-served demand (Equation (53)) should be added in the objective function. The penalty per unit of non-served electricity demand is set at 50 times the grid power price of the selected cities.

| ele_gent,solar+ele_gent,wind+bat_outt-bat_int+nsdt=ely_pt+dri_pt+drip_pt+eaf_pt | (52) |
| --- | --- |
| nsdc=tnsdt·PEN. | (53) |

Table S8 Fe content of materials.

| Material | Fe content | Source |
| --- | --- | --- |
| Iron ore DR pellet | 67% | 14 |
| Iron ore BF pellet | 65% | 14 |
| DRI | 92% | 15 |
| Pig iron | 96% | 16 |
| Scrap | 93% | 17 |
| Steel | 98% | 16 |

Table S9 Costs of iron ore and scrap. The iron ore pellet costs are the average costs across 2015~202014. Scrap costs are obtained with two scenarios based on the range of China’s historical scrap prices from 2010 to 202318.

| Material | Cost ($/t) |
| --- | --- |
| Iron ore DR pellet | 160 |
| Iron ore BF pellet | 135 |
| Scrap (low) | 286 |
| Scrap (high) | 392 |

Table S10 Stoichiometric data. Low heat value of natural gas and hydrogen are used. For H₂ preheating, electric heating is assumed to convert 50 m³ natural gas (88% heating efficiency) into power consumption (98% heating efficiency). Assume 100% Fe conversion rate for DR grade pellet to DRI.

|  | Value | Unit | Source |
| --- | --- | --- | --- |
| Power to hydrogen | 1.650 | MWh/MWh H₂ | 19 |
| Gas to DRI | 2.822 | MWh gas/t DRI | 20 |
| H₂ to DRI | 1.648 | MWh H₂/t DRI | 6 |
| Power to DRI (H₂ preheating) | 0.447 | MWh/t DRI | 6 |
| DR grade pellet to DRI | 1.373 | t/t DRI | Own calculations |
| Power to EAF (using DRI) | 0.785 | MWh/t Steel | 21,22 |
| Power to EAF (using scrap) | 0.699 | MWh/t Steel | 21,22 |
| Power to DRI (reheating) | 0.153 | MWh/t DRI | 23 |

Table S11 Cost data. The renewable technology-related data used in this study are specific to China. The electrolyzer and H₂ tank data have been validated by industry practice.

|  | CAPEX (2020) | CAPEX (2035) | Unit | OPEX | Lifetime | Source |
| --- | --- | --- | --- | --- | --- | --- |
| solar | 657 | 417 | $/kW | 1.4% | 25 | 24 |
| wind | 1086 | 662 | $/kW | 1.9% | 25 | 24 |
| battery power | 457 | 274 | $/kW | 5.0% | 15 | 24 |
| battery energy | 229 | 137 | $/kWh | / | 15 | 24 |
| electrolyzer | 400 | 232 | $/kW | 2.0% | 12 | 25 |
| H₂ tank | 394 | 394 | $/kg H₂ | 2.0% | 25 | 26 |
| DRI furnace | 370 | 370 | $/(t/y) | 2.0% | 30 | 21 |
| EAF | 296 | 296 | $/(t/y) | 2.0% | 30 | 21 |

Table S12 Technical data and boundaries. DRI related data is obtained from industry professionals. The reaction pressure in the DRI furnace is less than 2 bar27 and the outlet pressure of the alkaline electrolyzer could be up to 20 bar28. Therefore, the minimum SOC of the H₂ tank is approximated to be 10%, assuming a linear relationship between pressure and mass (ideal gas at constant volume and temperature).

|  | Value | Source |
| --- | --- | --- |
| Lithium-ion battery round-trip efficiency | 86% | 29 |
| Alkaline electrolyzer min load | 5% | 30 |
| Alkaline electrolyzer max load | 110% | 30 |
| H₂ tank min SOC | 10% | Own calculations |
| DRI min load | 70% | Midrex |
| DRI max load | 100% | Midrex |
| DRI load max ramp rate | 3.75% | Midrex |
| DRI max gas blend share | 30% | Midrex |
| DRI max gas/H₂ blend ramp rate | 2.5% | Midrex |

## 4. Input Data Structure

- Hourly renewable availability: 8760 hours, solar and wind, by region

- Energy and material costs: Use annuitized cost (including CAPEX and fixed OPEX)

- Stoichiometry parameters: efficiency, energy and material consumption

- Steel demand: Hourly and annual profile

## 5. Output Files and Interpretation

- Capacity variables: Sheet capacity. Installed capacities of electrolyzers, batteries, hydrogen tanks, etc.

- Operational variables: Sheet PowerBalance, H2_balance, DRI_balance, Material. Hourly dispatch of renewables, batteries, hydrogen use, furnace loads.

- Cost breakdown: Sheet cost. CAPEX, OPEX, energy costs, storage costs, total system cost. Sheet LCOX: levelized cost of commodities

- Performance metrics: Sheet UtilizationHours: annual utilization hours; Sheet curt: power curtailment rate.

- Emission: Sheet Emission: hourly CO2 emission